EFFECT OF DIVERSE INSERTS AND NANO FLUID ON THERMAL PERFORMANCE OF CONCENTRIC TUBE HEAT EXCHANGER HAVING SPIRAL TUBE

Aswathy M1, Ijas M S1, Saarthak Batra1, Shivasheesh Kaushik1

1Department of Mechanical Engineering, Birla Institute of Technology and Sciences, Pilani, Pilani Campus, Rajasthan, p20230094@pilani.bits-pilani.ac.in h20240094@pilani.bits-pilani.ac.in h20240135@pilani.bits-pilani.ac.in

ABSTRACT

A comparative study is presented to acknowledge the combined effect of diverse geometry of insert and fluid domain used in concentric tube exchanger (CTE). The combination of newly developed stable hybrid nanofluid with geometrical inserts enhances the overall surface area which provides a noble feature to this study compared with existing literature. Here, helical-shape, cubical-shape, cuboidal-shape, cylindrical-shape, conical-shape, and hemispherical-shape insert designs were considered for analysis. These were equipped over spiral tubes to analyse the thermal characteristics and its performance with diverse nanofluid. These fluids are developed through utrasonication technique and the combination strategy of diverse nanoparticle integration such as ZnO with Al2O3, and Al2O3 with CuO etc. at a size range of 10 nm and 0.05% volumetric fraction. The exchanger performance was examined under laminar-turbulent region for counter fluid flow situation. During experimentation the hot working fluid (H2O) moves in upper side of the exchanger shell. It is kept constant at 333 K with 4.02 L/min, whereas, the cold nanofluid is flow at ambient temperature of 304 K with a range of 0.72–2.95 L/min. The cold nanofluid moves inside the spiral tube. The geometrical and fluid domain were assessed for their impact on thermal performance, friction factor, Nusselt number, and effectiveness. The results reveals that the helical-shape and cubical-shape insert provides superior performance, it improves the thermal transfer rate by 38.5%, 25.2%, compared to the other inserts by 21.5%, 18.5%, 15.5% and 11.5%, respectively. Further, investigation has explored for nanofluids to examine the effect of these fluids inside the exchangers, installed with superior helical-shape insert. Al2O3 integrated CuO nanofluid presented the high and optimum thermal performance compared with other fluids.

Key Words: Concentric Tube Exchanger (CTE), Geometrical Domain, Fluid Domain, Thermal Behaviour, Exchanger Performance.